| Mission | TerraSAR-X/ Paz SAR | COSMO-SkyMed | Sentinal - 1 [C band] | ICEYE - X1 | ICEYE:X4-X32 |  | New |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Polarization | Single, Dual and Twin, | Single , Dual | Single and Dual | Single | Single (VV) |  |  |
| SAR mode | Spotlight (SL) Stripmap (SM) ,ScanSAR (SS) | Spotlight, Stripmap, ScanSAR | Spotlight, Stripmap, ScanSAR | Stripmap mode. | Spotlight, Strip map, ScanSAR. |  |  |
| Resolution | 1 m X 1m, 16 m x16, (Paz sar) 0.25 x 1 m | 3 m x 3m | 0.25 x 1, 2x2, 5x 20 | 3 x 3 | 1x3, 3x3 , 15x15 |  |  |
| Swath | 10 km(SL), 30 km (SM), 100 km (SS), HR Spot 5 km, Wide scan 200 km | 10 km for Spotlight, 30 km for Stripmap, 100 km for ScanSAR | Interferometric Wide Swath (IW): 250 km, 10 km SL, 30 km SM, 100 SS | stripmap 25 km | Scan mode 100, strip map 20 km, |  |  |
| Off nadir to access | 15 deg | 34 deg | 20 deg | up to 30 deg | up to 35 deg |  |  |
| Peak Power | 2260W | 13 KW | 15 KW | 3.2 Kw | 3.4 KW |  |  |
| Payload Mass | 394 kg | 1700 kg | 1000 kg | 61 Kg | 25 kg |  |  |
| Satellite Mass | 1230 kg | 2240 kg | 2000 kg | 85 kg | 70 kg |  |  |
| Chirp Bandwidth | 5 - 300 Mhz | 400 Mhz max | 100 Mhz | upto 300 Mhz | upto 400 Mhz |  |  |
| Pulse Rept Freq | 3000 - 6500 HZ | 3000 - 4500 HZ | 3000 - 4500 HZ | 2000- 10000 Hz | 2000-10000 Hz |  |  |
| Antenna | Active Phase Array | Phased array | Slotted Waveguide | Electronic | Electronic |  |  |
| Antenna size | 4.78 x 0.7 | 1.4 x 5.7 | 12.3 x 0.8 | 3.25 x 0.4 | 3.25 x 0.4 |  |  |
| System Noise | 5 db | NA | 3 db | NA | NA |  |  |
| Yaw Steering | Yes | Yes | Yes | yes | yes |  |  |
| Downlink | 300 Mbps | 520 MBps | 520 Mbps (X band) | > 100 Mbps | 140 Mbps |  |  |
| Accuracy pointing | 0.01 deg in elevation | 0.018 deg in elevation | 0.01 deg (each axis) | 0.01 | 0.005 |  |  |
| Local time | 18:00 hours | 6 .00 am Ascending | 18:00 ascending node | 6.00 am LTDN | NA |  |  |
| Altitude | 514 | 619.6 | 693 | 575 | 560 |  |  |
| Quantization | 8 bit to 16 bit Q | NA | FDBAQ, 10 bit | 16 bit | 10 bit |  |  |
| Repeat Cycle | 11 | 16 | 12 | 14.7 days | 22 |  |  |
| Launch Year/ Agency/Country | 2007 – Terra SAR 2018 – PAZ SAR DLR/Astrium | CSG-1: Dec 18, 2019 CSG-2: Jan 31, 2022 TASI/ASI | Sentinel-1A: April 3, 2014 ESA/TASI/Astrium/DLR | Jan12, 2018 Finland | Aug 2024 – Start 2019 Finland |  |  |
| Status | PAZ sar - Operational | Operational | Operational | Operational | Operational |  |  |
| Remarks |  |  |  |  |  |  |  |